Standard Operating Procedure (SOP) For Autonomous Vehicles in the High Bay Space

Purpose

The purpose of this SOP is to allow the IER lab to safely and effectively move vehicles in and out of the high bay space and perform programming/electronic work on them while in the space. This SOP will outline a specific order of operations for the team to follow to safely and effectively perform these operations.

Overview

The SOP is to ensure safe practices at all stages of the process. This includes moving the vehicles, exhaust fume considerations, and performing work on the vehicles. It will describe the procedures for moving the ballasts, opening the door, driving in, closing the door, performing work, and exiting the building.

Vehicles

- Lincoln Mkz hydrid electric vehicle equipped with a sensor suite designed to collect large-scale, real-world driving datasets for autonomous navigation and mapping tasks

- Yamaha Wolverine X-4 850 R-Spec 2022 Model off-road vehicle with added sensors and computers for autonomy

- Additional vehicles will be added as addenda to this procedure by IER, subject to review and approval by OARS and the Provost's Office

Prerequisites

All personnel involved in moving and working on the vehicles must receive the following training.

- Orientation for Labs, Makerspaces and Studios (SciShield)

- Fundamentals of Laboratory Safety (SciShield)

- Specific safety training appropriate to the space and tasks (PI)

These are University vehicles. All procedures in the Policy on Use of Vehicles for University Purposes must be followed. A designated spot in the high bay space must be taped out for each vehicle and be clear of all items, obstacles, and personnel before a vehicle is moved into the space. (Note: tape will be added the first time each vehicle arrives in the space as part of defining that space. We should have both vehicles move into the space at the same time the first time so that we can ensure adequate ventilation.) In addition, the following should always be available near the vehicle as well as inside the vehicle as well:

- An ABC or CO2 fire extinguisher rated for electrical and fuel fires should be on hand for entry, one for each vehicle, while the vehicle is in the space, and exit.

- A first aid kit must be present.

- Contact information for emergency services and team supervisors should be on hand (Location of this is TBD, once we get the vehicles in the space we will decide where a list will live for each, as different labs own the two different vehicles).

Drivers (these lists may be updated)

Only the following drivers can drive the Lincoln:

- TBD – waiting for the list from the lab

Only the following RIVeR members can drive the Yamaha:

- James Tukpah

- Dante Ben Cometto

- Aiden Reichenberg

- Tom Lutz

Procedures

- Moving the vehicles into the high bay space.

  - Inform the IER Lab Manager two days (48 hours) in advance of moving the vehicle. You must let the manager know which vehicle will be moving into the space. If the other vehicle is scheduled to be moving in or out during the same time, accommodations will need to be made. (Also, depending on where the vehicles end up parking, due to special constructions, it may not be possible to move the other in. This is TBD.)

  - The IER Lab Manager will notify Public Safety (James Witzgall or designated representative) and the OARS building manager(Brian Sullivan) of the move 1 day (24 hours) in advance.

  - Clear the area where the vehicle will traverse and park plus an additional three feet on all sides.

  - Shortly before the move, Public Safety (James Witzgall or designated representative) lowers ballasts in front of garage door, and opens the garage door. Public Safety will also ensure the sidewalk is clear for vehicle traversal. In the event that Public Safety is unavailable, the lab will employ the use of 3 additional researchers to verify the sidewalk is clear for vehicle traversal

  - An IER team member will observe from outside of the vehicle to ensure the path remains clear inside the building.

  - An IER team member will drive the vehicle into the garage, driving forward into the space with the vehicle exhaust facing the door. The vehicle will never be backed into or maneuvered within the high bay area. Each vehicle will have a designated spot. Order in which the vehicles can come into the building will be determined when the parking spots are assigned.

  - The vehicle engine will be turned off as soon as the vehicle has reached the designated spot. Ensure the vehicle is in park and the parking brake on (or equivalent).

  - Chock the wheels of the vehicle.

  - Clean and dry any water, snow, or ice on the high bay floor.

  - The door will be closed after an air exchange delay to be determined by monitoring carbon monoxide levels during an initial entry. (OARS will sample when each vehicle is moved into the space, and when both are present. We will have (3) sampling events to determine appropriate timing for the door. This SOP will be updated with the timing once it is determined.)

  - Public Safety (James Witzgall or designated representative) to replace ballasts.

- Working on either vehicle in the high bay

  - Disconnect the battery or power source before performing any work on the vehicle’s electrical system.

  - LOTO on electrical components (if necessary for Lincoln)

  - Work that may take place on the car includes installing, modifying, removing, and programming sensors, computers, and components. Electrical systems could be rated as high as 48V DC, but no higher.

  - Work that may not be performed includes work on the engine, fuel system, or related work.

  - Work that would require a specific, additional SOP includes changing tires and related work. Any such additional work must have an approved SOP in advance of the work.

  - The internal combustion engine shall not be started during work; it may only be run to enter and leave the building.

  - Disable autonomous features before working on sensors, controllers, or software systems, disable all autonomous functionalities

  - Standard safety procedures for the work to be performed should be followed at all times. Power tools require safety glasses and hair tied back, for example.

  - When working with electrical components, use tools with insulated handles to reduce the risk of electrical shock.

  - Keep the workspace clean and free of clutter. Ensure tools and equipment are organized to avoid tripping hazards.

  - Soldering should be performed in an appropriate fume capture device and not on the vehicle.

  - Verify that all power systems and circuits are within their rated capacities to prevent overheating or damage.

  - Double-check all wiring and connections for proper placement and insulation before powering the system.

  - Clearly label and document any system modifications or repairs in a system of record for dissemination to all team members.

  - Reference Appendix to see critical components and electrical specs.

Moving either vehicle out of the high bay space

  - Inform the IER Lab Manager two days (48 hours) in advance of moving the vehicle.

  - The IER Lab Manager will notify Public Safety and the OARS building manager of the move 1 day (24 hours) in advance.

  - Clear the area where the vehicle will traverse plus an additional three feet on all sides.

  - Shortly before the move, Public Safety (James Witzgall or designated representative)  will lower ballasts in front of garage door and open the garage door. Public Safety will also ensure the sidewalk is clear for vehicle traversal.

  - The vehicle will never be running with the garage door closed.

  - Remove chocks from vehicle.

  - Public safety will also assist with the vehicle entering the public road (Columbus Avenue).

  - An IER team member will observe from outside of the vehicle to ensure the path remains clear inside the building.

  - An IER team member will drive the vehicle into the garage, driving backward out of the space with the vehicle exhaust facing the door. The vehicle will never be turned around or maneuver within the high bay area. If both vehicles need to leave on the same day, an order will be determined as soon as the parking spots are determined. There may be a situation where if both vehicles are in the high bay that one cannot be moved without the other being moved, due to space.

  - The door will be closed after an air exchange delay to be determined by monitoring carbon monoxide levels during an initial entry. (OARS will sample when each vehicle is moved into the space, and when both are present. We will have (3) sampling events to determine appropriate timing for the door. This SOP will be updated with the timing once it is determined.)

  - Clean and dry any water, snow, or ice on the high bay floor.

  - Public Safety to replace ballasts.

Documentation – Appendices

Appendix A -- electrical components inventory for the Yamaha.

Appendix B -- Any student or faculty approved to work on any part of the vehicle or its systems shall be trained and sign off on this SOP for the Yamaha.

Appendix C -- All work performed on the vehicle shall be documented in a system of record. All entries and exits of the vehicle shall be similarly documented for the Yamaha.

Appendix D -- electrical components inventory for the Lincoln.

Appendix E -- Any student or faculty approved to work on any part of the vehicle or its systems shall be trained and sign off on this SOP for the Lincoln.

Appendix F -- All work performed on the vehicle shall be documented in a system of record. All entries and exits of the vehicle shall be similarly documented for the Lincoln.

Appendix A – Component List

# Component List

# Battery

Model → Lossigy LiFePO4 Lithium Battery Voltage → 48V

Current → 100A

Power → 5120Wh

# Kairos Autonomy Sub-System

## Kairos System

Model → Pronto4 Series 4 (P4S4) Voltage → 12V

Current → 5A

- 5A when not driving, or running actuators

- 20A running Actuators without engine

- 10A running actuators and engine

Power → 240W

# Hyper Spectral Camera Sub-System

## Allied Vision Alvium RGB Camera

Model → Alvium 1800 U-507  Voltage → 5V

Current → 0.4A

Power → 2.0W

## Short Wave Infrared (SWIR) HSI

Model → Imec SNAPSHOT SWIR 16 Bands  Voltage → 6V

Current → 3A

Power → 18W

## Visible Near Infrared (VNIR) HSI

Model → Ximea Snapshot VIS

Voltage → 5V

Current → 0.36A

Power → 1.8W

## Ouster OS1-64 Lidar

Model: OS1-64

Voltage → 24V

Current → 0.66667A

Power → 16W (Nominal)

## VNIR Spectrometer

Model → Pebble VNIR Voltage → 6V

Current → 0.1333A

Power → 0.8W

## NIR Spectrometer

Model → Pebble NIR  Voltage → 6V

Current → 0.1333A Power → 0.8W

## NUC Computer

Model →  Voltage → 19.5V

Current → 16.92A Power → 330 W

## Thermal Fan (Left Side Fan)

Model → Low-Voltage Equipment Cooling Fan  Voltage → 24V

Current → 0.73A Power → 17.52W

## Thermal Fan (Right Side Fan)

Model → Low-Voltage Equipment Cooling Fan

Voltage → 24V

Current → 0.73A

Power → 17.52 W

## NUC Buck Convertor

Model → Noyito 20 A Buck Converter Voltage → 24V

- Input Voltage range → 6V-40V

- Output Voltage Range → 1.25V-36V

Current → 20A

- Output Current range → 0.2A-20A

- Max Output Current → 20A

Power → 300W

## SWIR Buck Convertor

Model → Drok DC Buck Converter  Voltage →

- Input Voltage range → 5.3V-32V

- Output Voltage range → 1.2V-32V

Current → 8A

- Output Current → 8A

- Max Output Current → 12A

- Working current → 25mA = 0.025A

Power → 120W

# Hyper Drive Router Sub-System

# Router

Model → TP-Link Archer A7 V5

Voltage → 12V

Current → 1.5A

Power → 18W

Appendix B – Sign off

| Printed faculty/staff/student name | Date | Signature |
| --- | --- | --- |

Appendix C – Documented work on the vehicle

| Printed faculty/staff/student name | Date | Work being done on the car | Estimated time car will be in high bay |
| --- | --- | --- | --- |

Appendix D – Component List

Appendix E – Sign off

| Printed faculty/staff/student name | Date | Signature |
| --- | --- | --- |

Appendix F – Documented work on the vehicle

| Printed faculty/staff/student name | Date | Work being done on the car | Estimated time car will be in high bay |
| --- | --- | --- | --- |